The Tg8-PPC Driver User Guide.

0. Introduction

The Tg8 modules are fundamental components of the timing

system. We shall talk about of how to use them on the Power PC

based DSCs. As usually for any kind of equipment there is an

appropriate device driver that implements the interface

between the equipment and the operating system. The Tg8 driver

does such job for the famous Tg8 card. It allows the multiprocess

access to one or more Tg8 cards in parallel. Any process that wish

to interact with the Tg8 card must open a device. There is the

correspondence between the set of devices and the set of cards

installed on the given DSC. Namely, the first N devices (group 1)

link with the first Tg8 card (the value of N is 4 at the moment),

the next N devices (group 2) link with next Tg8 card, and so on.

A process can interact simultaneosly with a number of the Tg8 cards,

in this case it ought to open the same number of devices - one for

every card using appropriate groups of devices. A device being

opened reserved by its process until the process will close it

explicitly via the close function or implicitly by killing itself.

All processes that share the same Tg8 card (e.g. use devices from

the same group) have the equal rights in relation to that card;

nobody can reserve the Tg8 card for his own usage. Consequently

such processes should be rather carefull when they are going to

delete some resources related to the card - resources are shared!

I. Open the Tg8 device

A process uses the ordinar open function to obtain a device.

For example, fd = open("/dev/Tg8.1",O_RDWR,0). This function

returns an error (EBUSY) in the case if this device was reserved

by the other process, hence, a process should try to open the

next device from the same group.

II. Close the Tg8 device

The device is reserved by the process that opened it. That

process should execute the close function to free the corresponding

device. An example is close(fd), where the variable fd was

initialized by the open call. The driver disconnects given process

from any connected actions and cleans the VME bus interrupt flag

for any actions having no more processes being connected to them.

An application can specify additional actions, such as

Tg8ONCL_LEAVE   - leave the card in working (e.g. current) state;

Tg8ONCL_DISABLE - disable the card asynchronously;

Tg8ONCL_CLEAR   - disable the card asynchronously and clear the user table;

Tg8ONCL_DISABLE_SYNC - disable the card synchronously -

by means of the ioctl call with the function named Tg8DrvrON_CLOSE.

III. Write operation

The write function is used to declare the timing entity. This entity

includes the timing frame code (the wildcards are allowed for any

part of the frame except of the header byte), the control

word and the delay for decrementing counter. The control word specifies

the channel (or counter) number inside the range 1-8, the clock type

(the millisecond clock, the 10 MHz internal clock, the external #1 or #2)

that used by the counter, the counter start mode (normal, external start,

the chain) and the type of counter's output (to produce either the output

pulse, or the VME bus interrupt, or both, or nothing). In the last case

the timing entity will be disabled. The drivers looks for the same entity inside

its internal memory - so called user table - and if successed simply connect

the requesting process to existence timing entity. Otherwise it looks for the

first free entry in the user table and creates new timing entity. It

returns the error (ENOSPC) if the table is full.

An example is following:

#include <tg8drvr.h>

Tg8User act;

act.uEvent.Long = 0x21ff0101; /* Timing frame code with the wildcard */

act.uControl = 0x8000;        /* Do the VME bus interrupt only */

act.uDelay = 0;               /* No delay */

if (write(fd,&act,sizeof(Tg8User)) != sizeof(Tg8User)) {

printf("Can't write the timing entity\n");

exit(-1);

};

IV. Read operation

A process waiting for the events from the driver should use either

the read function call or the select function call or signals. In the last

case it must inform the driver (via the ioctl function call) about of what

signal number and the process identifier shall be used when event arrives

to the driver. In any case a process obtains event's details (including

the super cycle number, the occurence and ouput time, the frame code, the

current status of the firmware program) by means of the read function.

Below is the example of how to use the read function:

Tg8DrvrEvent event;

if (read(fd,&event,sizeof(Tg8DrvrEvent)) != sizeof(Tg8DrvrEvent)) {

perror("ReadEvent");

exit(-1);

};

/* Display event's details and so on ... */

V. IO Control functions

The most of functions work with the union declared as Tg8IoBlock iob.

It contains structures used by the appropriate IO Control functions.

Function		Description and an example

--------		--------------------------

------------------------------------------------------------------------

0. Functions related to the Tg8 driver

------------------------------------------------------------------------

Tg8DrvrDEBUG_ON          Allows to logg the debugging printout on the console.

ioctl(fd,Tg8DrvrDEBUG_ON,NULL);

Tg8DrvrDEBUG_OFF         Stops the logg.

ioctl(fd,Tg8DrvrDEBUG_OFF,NULL);

Tg8DrvrGET_DRI_VERSION   Get the driver's revision date.

ioctl(fd,Tg8DrvrGET_DRI_VERSION,iob.Ver);

Tg8DrvrGET_STATUS        Get the driver status and alarm code.

ioctl(fd,Tg8DrvrGET_STATUS,&iob.Status);

Tg8DrvrGET_CONFIGURATION Get the current driver's configuration.

ioctl(fd,Tg8DrvrGET_CONFIGURATION,&iob.GetConfig);

Tg8DrvrINSTALL_MODULE    Install the new module dynamically.

iob.InsModule.Index = 3; /* The third module */

iob.InsModule.Address.VMEAddress = 0xDEC200000;

iob.InsModule.Address.InterruptVector = 0xBA;

iob.InsModule.Address.InterruptLevel = 2; /* Always 2 */

iob.InsModule.Address.SwitchSettings = 0; /* 10 MHz clock */

iob.InsModule.Address.SscHeader = 0x20;

ioctl(fd,Tg8DrvrINSTALL_MODULE,&iob.InsModule);

------------------------------------------------------------------------

1. Functions related to the Tg8 module (card)

------------------------------------------------------------------------

Tg8DrvrENABLE_MODULE    Enable outputs and interrupts asynchronously.

ioctl(fd,Tg8DrvrENABLE_MODULE,NULL);

Tg8DrvrENABLE_SYNC      Enable outputs and interrupts synchronously with the SSC frame.

ioctl(fd,Tg8DrvrENABLE_SYNC,NULL);

Tg8DrvrDISABLE_MODULE   Disable outputs and interrupts asynchonously.

ioctl(fd,Tg8DrvrDISABLE_MODULE,NULL);

Tg8DrvrDISABLE_SYNC     Disable outputs and interrupts synchronously with the SSC frame.

ioctl(fd,Tg8DrvrDISABLE_SYNC,NULL);

Tg8DrvrRESET_MODULE     Hardware reset of the card.

ioctl(fd,Tg8DrvrRESET_MODULE,NULL);

Tg8DrvrSIMULATE_PULSE   Simulate a set of pulses on the card. The card should be disabled.

iob.SimPulseMask =0x03; /* Use counters 1 and 2 */

ioctl(fd,Tg8DrvrSIMULATE_PULSE,&iob);

------------------------------------------------------------------------

Tg8DrvrDATE_TIME	    Get the current date and time.

ioctl(fd,Tg8DrvrDATE_TIME,&iob.DateTime);

Tg8DrvrSC_INFO   	    Get the most important Super Cycle information.

ioctl(fd,Tg8DrvrSC_INFO,&iob.ScInfo);

Tg8DrvrSET_SSC_HEADER    Set the SSC Event Header code.

iob.SscHeader = 0x20; /* The SPS SSC event header code */

ioctl(fd,Tg8DrvrSET_SSC_HEADER,&iob.SscHeader);

------------------------------------------------------------------------

Tg8DrvrUSER_TABLE       Read a part of the user table back from the card RAM.

Tg8IoBlock iob;

iob.Range.Row = 5;  /* The first action number */

iob.Range.Cnt = 10; /* Number of actions to read */

ioctl(fd,Tg8DrvrUSER_TABLE,&iob.UserTable);

Tg8DrvrRECORDING_TABLE  Get a part of the Recording table.

iob.Range.Row = 5;  /* The first action number */

iob.Range.Cnt = 10; /* Number of actions to read */

ioctl(fd,Tg8DrvrRECORDING_TABLE,&iob.RecTable);

Tg8DrvrHISTORY_TABLE    Get a part of the Event History Table.

iob.HistTable.Cnt = 1000; /*Number of the last entries to read */

ioctl(fd,Tg8DrvrHISTORY_TABLE,&iob.HistTable);

Tg8DrvrCLOCK_TABLE      Get a part of the Clock table.

iob.ClockTable.Cnt = 15; /*Number of the last entries to read */

ioctl(fd,Tg8DrvrCLOCK_TABLE,&iob.ClockTable);

Tg8DrvrINTERRUPT_TABLE  Get a part of the Interrupt table.

Tg8IoBlock iob;

iob.Range.Row = 1;  /* The first entry number */

iob.Range.Cnt = 16; /* Number of aentries to read */

ioctl(fd,Tg8DrvrINTERRUPT_TABLE,&iob.IntTable);

Tg8DrvrAUX_TABLE	   Get the Auxiliary table.

ioctl(fd,Tg8DrvrAUX_TABLE,&iob.AuxTable);

------------------------------------------------------------------------

Tg8DrvrGET_DPRAM       Read the whole DPRAM content.

ioctl(fd,Tg8DrvrGET_DPRAM,&iob.Dpram);

Tg8DrvrGET_RAW_STATUS  Read current status directly from the DPRAM.

ioctl(fd,Tg8DrvrGET_RAW_STATUS,&iob.StatusBlock);

Tg8DrvrTELEGRAM	  Read a telegram buffer for selected accelerator machine.

iob.Telegram.Machine = Tg8CPS;

ioctl(fd,Tg8DrvrTELEGRAM,&iob.Telegram);

------------------------------------------------------------------------

Tg8DrvrGET_FIRMWARE_VERSION  Get the firmware program revision date.

ioctl(fd,Tg8DrvrGET_FIRMWARE_VERSION,iob.Ver);

Tg8DrvrTRACE_FIRMWARE   Get the firmware trace data.

ioctl(fd,Tg8DrvrTRACE_FIRMWARE,&iob.Trace);

Tg8DrvrRELOAD_FIRMWARE  Reset the card and restart the firmware program.

ioctl(fd,Tg8DrvrRELOAD_FIRMWARE,&iob.FirmwareObject);

Tg8DrvrRELOAD_ACTIONS   Reload the Tg8 module by the driver's user table.

ioctl(fd,Tg8DrvrRELOAD_ACTIONS,NULL);

------------------------------------------------------------------------

2. Functions related to process connections

------------------------------------------------------------------------

Tg8DrvrON_CLOSE       Predefine the device close mode.

iob.CloseMode.Mode = Tg8ONCL_CLEAR; /* Disable the card and

clear the user table */

iob.CloseMode.SimPulseMask = 0x01;  /* And simulate the pulse #1 */

ioctl(fd,Tg8DrvrON_CLOSE,&iob.CloseMode);

Tg8DrvrSET_SIGNAL     Set the Pid & Signal codes.

iob.Signal.Pid = getpid();

iob.Signal.Signal = sig_num;

ioctl(fd,Tg8DrvrSET_SIGNAL,&iob.Signal);

Tg8DrvrGET_SIGNAL     Read Pid & Signal codes back.

ioctl(fd,Tg8DrvrGET_SIGNAL,&iob.Signal);

------------------------------------------------------------------------

Tg8DrvrWAIT_EVENT    Wait for the given event (no wildcards allowed).

iob.Event.Event.Long = 0x211E0101;

ioctl(fd,Tg8DrvrWAIT_EVENT,&iob.Event);

Tg8DrvrFILTER_EVENT  Filter events in the user table

(connect to the subset of actions from the user table).

iob.Filter.Event.Long = 0x21FFFFFF;

ioctl(fd,Tg8DrvrFILTER_EVENT,&iob.Filter);

------------------------------------------------------------------------

Tg8DrvrGET_QUEUE_LENGTH  Get the events queue length.

ioctl(fd,Tg8DrvrGET_QUEUE_LENGTH,&iob.QueueLength);

Tg8DrvrPURGE_QUEUE       Discard all events on the process's queue.

ioctl(fd,Tg8DrvrPURGE_QUEUE,NULL);

------------------------------------------------------------------------

3. Functions related to the timing actions

------------------------------------------------------------------------

Tg8DrvrSET_ACTION_STATE  Set the actions Enabled/Disabled state.

iob.Range.Row = 10; /* The first action number */

iob.Range.Cnt = 2;  /* The number of actions */

iob.ActState.State = Tg8ENABLED; /* Enable them */

ioctl(fd,Tg8DrvrSET_ACTION_STATE,&iob.ActState);

Tg8DrvrCLEAR_ACTION      Clear the range of the actions.

iob.Range.Row = 5;  /* The first action number to be cleared */

iob.Range.Cnt = 10; /* The number of actions to clear */

ioctl(fd,Tg8DrvrCLEAR_ACTION,&iob.Range);

------------------------------------------------------------------------

4. Functions related to the timing objects

------------------------------------------------------------------------

Tg8DrvrSET_OBJECT    Define the new timing object.

Tg8Object *obj = &iob.Object;

obj->Id = 1001;          /* Object's identifier */

obj->Timing.Machine = 0; /* No gate conditions */

obj->Timing.Trigger.Long = 0x02FFFFFF; /* Trigger code */

obj->Timing.LineCw[0] = 0x4100; /* Use the counter #1 */

obj->Timing.LineDelay[0] = 120; /*     and delay 120 ms */

ioctl(fd,Tg8DrvrSET_OBJECT,obj);

Tg8DrvrREMOVE_OBJECT  Remove the timing object.

iob.ObjectId = 1001;         /* Object's identifier */

ioctl(fd,Tg8DrvrREMOVE_OBJECT,&iob.ObjectId);

Tg8DrvrOBJECTS_LIST   Get the list of all known timing object IDs on the module.

iob.ObjectsList.Length = 256;  /* Maximal number of IDs */

ioctl(fd,Tg8DrvrOBJECTS_LIST,&iob.ObjectList);

------------------------------------------------------------------------

Tg8DrvrGET_OBJECT     Get the timing object's details.

iob.ObjectDescr.Object.Id = 1001;

ioctl(fd,Tg8DrvrGET_OBJECT,&iob.ObjectDescr);

Tg8DrvrOBJECT_PARAM   Change the timing object's parameter.

iob.ObjectParam.Id = 1001;

iob.ObjectParam.Sel= Tg8SEL_DELAY; /* Change the delay */

iob.ObjectParam.Value = 17;  /* New value for the delay */

iob.ObjectParam.Aux = 0;     /* Donn't change clock type */

ioctl(fd,Tg8DrvrOBJECT_PARAM,&iob.ObjectParam);

------------------------------------------------------------------------

Tg8DrvrCONNECT        Connect to the timing object.

iob.ObjectConnect.Id = 1001;

iob.ObjectConnect.Mask = 1; /*1=Connect; 0=Disconnect */

ioctl(fd,Tg8DrvrCONNECT,&iob.ObjectConnect);

------------------------------------------------------------------------

VI. The Driver installation procedure

The installation procedure scans the range of possible addresses (0xC00000 -

0xC10000),

reads and check its interrupt vector and the clock switch setting, reads

the card hardware status, sets the SSC event header code and finally installs

the interrupt service routine and unmasks interrupt level according to the

driver configuration parameters.

VII. The Driver uninstallation procedure

The uninstallation procedure resets each configured module, clears appropriate

interrupt vector and frees the memory allocated during the installation.

VIII. The restrictions

The number of actions in the user table is 255, the first action is referenced

by the number 1. Any number of actions can share the same counter inside the Tg8

card. Any actions that use only the VME bus interrupt flag (without of the pulse

generation) have lower priority (in the firmware) in comparison with actions

producing pulses; their delay value is ALWAYS ZERO even if a process tries to

setup it to nonzero. The firmware program uses the DPRAM interrupt for all such

actions and NEVER uses so called direct actions (the Tg8 hardware allows to reuse

its counters while they are counting in the case if new action will use zero delay).

These actions can even produce an interrupt later than the next millisecond frame

will arrive (for heavy loaded firmware program).

*******************************

APPENDIX A. The data structures

*******************************

-- The User Table --

Tg8User Table[Tg8ACTIONS]; /* Array of actions */

where the Tg8User structure has a format:

Tg8Event	uEvent;   /* 32 bits Trigger event */

Word		uControl; /* 16 bits Control word  */

Word         uDelay;   /* 16 bits Delay value */

-- The Recording Table

Tg8Recording Table[Tg8ACTIONS]; /* Array of Recording blocks */

where

Tg8Recording structure has a format:

Uint   rSc;        /* The 32 bits Super Cycle number (only 24 bits are meaningfull) */

Uint   rOcc,       /* Occurence time from the start of the last S-Cycle */

rOut;       /* Output time from the start of the last S-Cycle */

Word   rNumOfTrig; /* How many times an action was triggered since the last reset */

Byte   rOvwrCnt,   /* Number of the lost VME bus interrupts */

rOvwrAct;   /* What action was in the interrupt table waiting for the driver service */

-- The Interrupt Table --

Tg8Interrupt CntInter[Tg8CHANNELS]; /* Counter interrupts description list */

Tg8Interrupt ImmInter[Tg8IMM_INTS]; /* Immediate interrupts description list */

where

Tg8CHANNELS (the number of channels, or counters) is equal to 8,

Tg8IMM_INTS (the number of so called immediate actions, e.g. actions that use the

VME bus interrupt only and being triggered at the same millisecond),

Tg8Interrupt is a structure with the following format:

Tg8Event iEvent;     /* The trigger event frame */

Uint     iSc;        /* The Super Cycle number */

Uint     iOcc,       /* Occurence time from the start of the last S-Cycle */

iOut;       /* Output time from the start of the last S-Cycle */

Tg8IntAction iExt;   /* Extra data concerning the interrupted action */

where

Tg8IntAction organized as follows:

Byte     iRcvErr;    /* Value of the reception error register */

Byte     iOvwrAct;   /* Which action was overwitten (if not 0) -- DONN'T USED */

Byte     iAct;       /* Which action was triggered and fired */

Byte     iSem;       /* Semaphore to solve the read/write conflicts */

-- The History Table --

Tg8History Table[Tg8HISTORIES]; /* Histories array */

where

Tg8HISTORIES is 1000 (the number of entries inside of the Tg8 card RAM),

Tg8History formatted as follows:

Tg8Event hEvent;     /* Received event frame code */

Uint     hSc;        /* The Super Cycle number */

Uint     hOcc;       /* Occurence time from the start of the last S-Cycle */

Byte     hRcvErr,    /* Value of the reception error register */

hHour,      /* The current hour, */

hMinute,    /* minute, */

hSecond;    /* second */

-- The Clock Table --

Tg8Clock Table[Tg8CLOCKS]; /* Clock errors info records */

where

Tg8CLOCKS is 16 (the table capacity inside of the Tg8 RAM),

Tg8Clock is a structure with the format:

Tg8Event cMsEvent;   /* The last valid ms frame code */

Uint     cNumOfMs;   /* Number of previous ms frames received without of errors */

Uint     cSc;        /* The Super Cycle number */

Uint     cOcc;       /* Occurence time from the start of the last S-Cycle */

Byte     cRcvErr,    /* Value of the reception error register */

cHour,      /* The current hour, */

cMinute,    /* minute, */

cSecond;    /* second */

-- The Auxiliary Table --

Tg8Event aEvent;     /* The last received event frame */

Tg8Event aMsEvent;   /* The last received ms frame */

Uint     aScLen;     /* The last completed S-Cycle length */

Uint     aScTime;    /* The current value of the S-Cycle time */

Uint     aSc;        /* The Super Cycle number */

Uint     aNumOfSc;   /* Number of SC received since the last Reset */

Word     aTrace;     /* Used for the firmware debugging */

Word     aIntSrc;    /* There are 16 interrupt sources each

corresponds to a bit in this mask */

Word     aNumOfBus,  /* Number of the MPC bus interrupts */

aNumOfSpur; /* Number of the spurios interrupts */

Word     aMbox;      /* The last mailbox operation code */

short    aCoco;      /* The last mailbox operation completion code */

Word     aFwStat;    /* The firmware state (booting,runnung,stopped) */

Word     aSscHeader; /* The SSC header selected (SPS or LEP) */

Word     aNumOfEv,   /* Number of events received in the current S-Cycle */

aPrNumOfEv; /* Number of events received in the previous S-Cycle */

Word     aNumOfAct;  /* Number of defined actions */

Word     aAlarms;    /* The bitmask of error conditions */

Tg8DateTime aDt;     /* Date, time, rcv error */

Byte     aFwVer[16]; /* The firmware version */

Word     aSem;       /* Semaphore to solve the read/write conflicts */

Word     aQueueSize;   /* Number of frames in the AT queue */

Word     aMaxQueueSize;/* Maximal queue size */

Word     aMovedFrames; /* Number of unserved frames in the AT queue at the next MS */

Uint     aMovedScTime; /* The S-Cycle time when frame moving detected */

Uint     aMovedSc;     /* and the Super Cycle number */

Word     aProcFrames,  /* Number of the frames processed during the last MS */

aModStatus;   /* The module's status  */

Tg8Event aQueue[8];    /* Incoming frames queue */

Word     aTelegLEP [Tg8GROUPS]; /* The current LEP telegram */

Word     aTelegSPS [Tg8GROUPS]; /* The current SPS telegram */

Word     aTelegCPS [Tg8GROUPS]; /* The current CPS telegram */

Word     aTelegPSB [Tg8GROUPS]; /* The current PSB telegram */

Word     aTelegLPI [Tg8GROUPS]; /* The current LPI telegram */

where

Tg8GROUPS is 24 (the number of groups in the PS telegrams),

Tg8DateTime structured as follows:

Byte     aYear,      /* The date part */

aMonth,

aDay,

aSpare1;    /* 32-bits alignment is required */

Byte     aRcvErr,    /* Value of the reception error register */

aHour,      /* The time part */

aMinute,

aSecond;

-- Event -- Tg8DrvrEvent

Incoming timing event (as returned by the read function) has following format:

int          Id;     /* Timing object identifier OR action number */

Tg8Event     Event;  /* The trigger value as declared in the user table */

Tg8Interrupt Inter;  /* Interrupt information about of this event */

Word	        Alarms; /* The firmware alarms bit mask ORed with the

driver alarms bit mask */

Word         FwStat; /* The current status of the Firmware Program */

where

The firmware status bit masks:

Tg8FS_RUNNING  = 0x1,   /* The firmware program is running normally */

Tg8FS_ALARMS   = 0x2,   /* Some errors (alarms) were occured during the

execution of event program */

and Alarms bit masks:

Tg8ALARM_OK       = 0,	/* No alarms */

The Firmware program sets following bits:

Tg8ALARM_LOST_IMM = 0x2,     /* The immediate action's interrupt was lost */

Tg8ALARM_LOST_OUT = 0x4,     /* The counter pulse was lost */

Tg8ALARM_MANY_PROC= 0x8,     /* The AT processes queue is full */

Tg8ALARM_QUEUED_PROC = 0x10, /* The AT process was queued (as usually!) */

Tg8ALARM_DIFF_EVN = 0x20,    /* The number of events for 2 seq. SCs are different */

Tg8ALARM_DIFF_LEN = 0x40,    /* The SC length has been changed */

Tg8ALARM_MOVED_PROC=0x80,    /* The AT process is not completed before the next ms */

Tg8ALARM_MOVED_IMM =0x100,   /* The immediate interrupts where moved */

Tg8ALARM_ACT_DISBL =0x200,   /* The action was disabled, but till is in the CAM */

Tg8ALARM_IMMQ_OVF  =0x400,   /* Immediate actions queue overflowed */

Tg8ALARM_MBX_BUSY  =0x800,   /* The mailbox busy, but there is the new request */

The Tg8 Driver sets the rest bits:

Tg8ALARM_UNCOM    = 0x1000,  /* A counter gives the VME interrupt without the

appropriate data in the Interrupt Table */

Tg8ALARM_BAD_SWITCH=0x2000,  /* The switch ExtClock/10MHz is in bad position */

Tg8ALARM_INT_LOST  =0x4000   /* Interrupt info was lost due to the driver is too busy */

========================================================================================